Customer Product Quality Requirements Form

Part Number: OXW006		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXW006	o-Phenanthroline 4 GPL Aqueous Solution

Refer to each raw material SDS before handling the material for the first time.

Wear gloves (Nitrile), and goggles during production.

Rinse vessel with deionized water (9765)  before use.

Inspect stir bar and remove any traces of metal before use.

1 Liter formulation approximate

For each liter of final solution dissolve 4.00 grams of 1,10-Phenanthroline Monohydrate Reagent ACS (P1938) into 800 mls of deionized water (9765). Add 1 ml of Hydrochloric Acid Reagent ACS 36.5-38% (H2505) to help the Phenanthroline (P1938) dissolve. Dilute to 1 liter with deionized water (9765). Stir for a couple hours until completely dissolved.

*Note: Make sure solution in clear water white before release

Use the following equation to verify 4.00 +- 0.02 gpL phenanthroline monohydrate:

gpL = (act gm P1938 / final volume in liters)

Revision:  0.0                           Date: 11/14/2019	Initial: STN

Expiration: 1 Year

Special posting instructions:  Put calculated gpl on CoA to 2 decimal places.

Special Labeling Instructions: Do  not put calculated value on label

Special Packaging:   Wear gloves (Nitrile) and goggles

Special Filtering Instructions: do not filter if debris present, solution needs to stir longer.

Packaging:  1 Liter amber glass

Retain: amber glass

HMIS CODE: 100B

Packager Personal Protection: Gloves, goggles

Technician Personal Protection:  Gloves, goggles

Standardization: use densitometer to verify density. Use above equation to verify GPL of 4.00±0.02 phenanthroline monohydrate

Appearance / Odor: Clear water white liquid   / odorless           Initial :_________

Disposal:  General waste

Revision 0.0 11/14/19: Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /