Customer Product Quality Requirements Form

Part Number: 3845		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

3845		Ferrous Ammonium Sulfate

0.5 Normal Certified Solution

CLEAN THE GLASSWARE OUT WITHDEIONIZED WATER (9765) BEFORE STARTING BATCH.

WEAR GLOVES (AT LEAST ONE OF THE CHEMICALS IN THIS PRODUCT WILL PERMEATE NITRILE GLOVES. YOU MUST WEAR A BARRIER GLOVE UNDER A LARGER NITRILE GLOVE. REMEMBER, THE BARRIER GLOVE IS NOT A DISPOSABLE GLOVE.) AND GOGGLES WHEN MAKING THIS.

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE 196.07 GRAMS OF REAGENT ACS GRADE FERROUS AMMONIUM SULFATE * 6H20 ( F3002) INTO 800 MLS DEIONIZED WATER (9765). AFTER ALL OF THE FERROUS AMMONIUM SULFATE ( F3002) HAS DISSOLVED, SLOWLY, ADD 40 MLS. OF CONCENTRATED REAGENT ACS SULFURIC ACID ( S9191) DILUTE TO 1 LITER. WITH DEIONIZED WATER STIR UNTIL HOMOGENOUS.

REVISION:  0.0	    DATE: 11/14/2019                     INITIAL:STN

EXPIRATION: 6 MONTHS

Special Posting Instructions: Put normality on CoA to 4 decimal places.

Special Labeling Instructions: Put normality on label to 4 decimal places. (0.4995-0.5005)

SPECIAL PACKAGING INSTRUCTIONS:  NONE

PACKAGING: clear plastic

Retain: Clear Plastic

HMIS CODE: 300C

PERSONAL PROTECTION: GLOVES(AT LEAST ONE OF THE CHEMICALS IN THIS PRODUCT

WILL PERMEATE NITRILE GLOVES. YOU MUST WEAR A BARRIER GLOVE UNDER A LARGER

NITRILE GLOVE. REMEMBER, THE BARRIER GLOVE IS NOT A DISPOSABLE GLOVE.),GOGGLES

STANDARDIZATION: QCSP3 TO 0.5000 +- 0.0005 @ 20° C NORMAL

APPEARANCE OF FINAL SOLN/ ODOR: CLEAR LIGHT GREEN/NONE   INITIALS:___________

DISPOSAL:  GENERAL

REVISED 0.0, 11/14/19: Original batch card creation. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /