Customer Product Quality Requirements Form

Part Number: OXY537		    X      New Product       	               Revised Product

Company: OXY			Contact:  Dale Jones

Address:

Phone:	316-529-7347			email: Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXY537	         Phosphoric Acid 1:1-Sulfuric Acid 1:2 Mixed Acid

Refer to each raw material SDS before handling the material for the first time.

Wear gloves (Nitrile), and goggles during production.

Rinse vessel with deionized water (9765)  before use.

Inspect stir bar and remove any traces of metal before use.

*Customer may order 100 bottles a year.

1 Liter formulation approximate

For each liter of final solution add 500 mls of phosphoric acid 1:1 (6870) to vessel.

Next, for each liter of final solution, add 500 mls of sulfuric acid 1:2 (VUL039F) to the phosphoric acid 1:1 solution and stir until homogenous.  Do not dilute further. The two solutions may combine and be just short of 1 liter. This is OK.

Revision:  0.0		Date: 11/04/2019		initial: STN

Expiration:    2 years

Special Posting Instructions: None

Special labeling instructions: None

Filtering instructions:  can be glass wool filtered but y strainer if made in a tank

Special Packaging Instructions:  wear gloves and goggles, this solution is corrosive!

Packaging:   2.5 liter Clear glass acid bottle (PVC)

Retain: Clear plastic

HMIS CODE:   300C, corrosive liquid

Personal Protection:  Gloves, goggles

Standardization:   None

Appearance of Final Soln/ Odor:   clear water white/ odorless  ____Init:

Disposal:   general

Revision 0.0: 11/04/2019, Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /