Customer Product Quality Requirements Form

Part Number: VUL355C		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL355	BROMOTHYMOL BLUE 0.1% W/V IN MEOH/WATER 90:10

pH = 7.5

## Rinse vessel with deionized water before use

Inspect stir bar and remove any traces of metal before use

Wear gloves (At least one of the chemicals in this product will permeate nitrile gloves. You must wear a barrier glove under a larger nitrile glove. Remember, the barrier glove is not a disposable glove.) and goggles when making this batch.

1 Liter formulation

For each liter of final solution, dissolve 1.00 grams of Reagent ACS grade Bromothymol blue (B7112) into exactly 100 mls of reagent ACS grade Methanol (M2515). Stir until all of the bromothymol blue dissolves. Dilute to 1 liter with deionized water (9765) and stir until homogenous. Next, adjust the pH of the solution to 7.50 +- 0.05 with 0.1 normal sodium hydroxide (8625) or 0.1 normal hydrochloric acid (4130). Make sure pH is stable before removing pH probes. The color of the solution should be green-blue.

Use the following equation to verify a 0.10 +- 0.01% w/v bromothymol blue solution:

w/v% = (Act gm B7112 / final volume in liters / 10)

Revision:  0.3                           Date: 12-19-2018	            Initial: STN

Expiration: 1 Year

Special posting instructions: put pH (3 decimal places) and w/v% bromothymol blue to 2 decimal places on COA.

Special labeling instructions: do not put actual pH or calculated w/v% on label.

Special filtering instructions: should not need filtered but can be glass wool or paper filtered if needed.

Special Packaging: Wear gloves and goggles

Packaging: VUL355 = 500 ml clear plastic VUL355A = 4 liter cube VUL355B = 100 ml clear plastic,VUL355C= 10 Liter Cube

HMIS CODE: 200C

Personal Protection:  Gloves, goggles

Standardization:  Adjust pH to 7.50 +- 0.05 with 0.1 normal sodium hydroxide or 0.1 normal hydrochloric acid.Density to 0.98522 ± 0.00062 @ 20°C (0.98460 - 0.98584) Use above equation to verify a 0.10 +- 0.01% w/v bromothymol blue solution

Appearance / Odor: Clear green-blue liquid / Alcohol odor Initial: _________

Disposal:  organic waste

Revision 0.0 09/11/06 Creation date for batch card. STN

Revision 0.1, 06/05/07: Added pH requirement as per Vulcan Geismar. STN

Revision 0.2, 10/23/15: Added density specifications. DEJ

Revision 0.3, 12-19-2018: added verification calculation. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /