Customer Product Quality Requirements Form

Part Number: VUL359A		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL359	HYDROCHLORIC ACID 3.06 MOLAR

Rinse vessel with deionized water before use.

Inspect stir bar and remove any traces of metal before use

Wear gloves and goggles when making this batch.

Note: wear respirator or handle the hydrochloric acid under the hood.

1 Liter formulation

For each liter of final solution, slowly add 253 mls of Reagent ACS grade Hydrochloric Acid (H2505) to about 700 mls of deionized water (9765) and stir until homogenous. Once cooled down to room temperature, dilute to 1 liter with deionized water and stir until homogenous.

Use the following equation to verify a nominal 3.06 +- 0.02 molar hydrochloric acid solution:

Molarity = (Act mls H2505 / 82.68 / Final volume in liters)

Revision:  0.2		 Date: 11-07-2019		Initial: STN

Expiration: 2 Years

Special Posting Instructions: Put calculated molarity to 2 decimal places on COA

Special Labeling Instructions: Do not put calculated molarity on label.

Special Packaging: Wear gloves and goggles, this solution is very corrosive!

Packaging: VUL359 = 4 Liter Cube, VUL359A = 10 Liter Cube, VUL359B = 4 liter clear plastic

HMIS CODE: 300C, Corrosive liquid

Personal Protection:  Gloves, goggles

Standardization:  Use above calculation to verify a nominal 3.06 +- 0.02 Molar Hydrochloric acid solution. Use densitometer to verify density of 1.05030 +/- 0.00070 @ 20°C

Appearance / Odor: Clear liquid, odorless    Initial:_________

Disposal:  general

Revision 0.0, 09/13/06: Creation date for batch card. STN

Revision 0.1, 10/23/15: Added density specifications. DEJ

Revision 0.2, 11-07-2019: added verification calculation and removed standardization as this solution is not used as a titrant. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /