Customer Product Quality Requirements Form

Part Number: SYN018A		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

SYN018	TARTARIC ACID 20% W/V SOLUTION

NOTE: rinse vessel with deionized water before use.

Inspect stir bar and remove any traces of metal before use

Wear gloves and glasses when making this batch.

1 liter formulation

For each liter of final solution dissolve 200.0 grams of Reagent ACS grade tartaric acid (T2600) into about 800 mls of deionized water (9765). Once all of the solids dissolve, dilute to 1 liter with deionized water and stir until homogenous.

Use the following equation to verify a 20.0 +- 0.1% w/v zinc chloride:

w/v% = (Act gm T2600 / final volume in liters / 10)

Revision:  0.1                           Date: 12-19-2018	Initial: STN

Expiration: 1 Year

Special posting instructions: put calculated w/v% tartaric acid to 1 decimal place on COA.

Special labeling instructions: do not put calculated value on label.

Special filtering: Can be glass wool, paper or y-strainer filtered if needed.

Special Packaging Instructions:  wear gloves and goggles

Packaging:  4 liter cube

HMIS CODE:  200C

Personal Protection:  Gloves, goggles

Standardization: Use above equation to verify a 20.0 +- 0.1% w/v zinc chloride

Appearance Of Final Soln/ Odor: Clear water white odorless liquid

Disposal:  general

Revision 0.0 03/10/04 Creation date for batch card. STN

Revision 0.1, 12-19-2018: added verification calculation. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /