Customer Product Quality Requirements Form

Part Number: VUL035A		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL035                           SODIUM SULFITE

### 2 NORMAL

RINSE VESSEL WITH DEIONIZED WATER (9765) BEFORE USE.

WEAR GLOVES AND GOGGLES WHILE MAKING THIS BATCH.

1-LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE 126.04 GRAMS OF REAGENT ACS GRADE SODIUM SULFITE (S3703) INTO ABOUT 500 MLS OF DEIONIZED WATER (9765).  ONCE COMPLETELY DISSOLVED, DILUTE TO 1 LITER WITH DEIONIZEDWATER (9765) AND STIR UTNIL HOMOGENOUS.

REVISION:  0.0                     DATE:  01/12/01                     INITIAL:___________.

EXPIRATION:   6 MONTHS

SPECIAL PACKAGING INSTRUCTIONS:  NONE

PACKAGING:   VUL035 = 4 LITER CUBE ,VUL035A=10 Liter Cube VUL035B = 4 LITER CLEAR PLASTIC

HMIS CODE :   200C

PERSONAL PROTECTION:  GLOVES, GOGGLES, LABCOAT

STANDARDIZATION: NONE

APPEARANCE OF FINAL SOLN/ ODOR:    CLEAR WATER WHITE / ODORLESS

DISPOSAL:   GENERAL

REVISION 0.0 01/12/01: CREATION DATE FOR BATCH CARD.  MG

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /