Customer Product Quality Requirements Form

Part Number: OXW015		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXW015	Barium Chloride 10% w/v Neutralized

Refer to each raw material SDS before handling the material for the first time.

Wear gloves (Nitrile), and goggles during production.

Rinse vessel with deionized water (9765)  before use.

Inspect stir bar and remove any traces of metal before use.  .

1 Liter formulation approximate

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE 100.0 GRAMS OF REAGENT ACS GRADE

BARIUM CHLORIDE DIHYDRATE (B0360) INTO ABOUT 700 MLS OF DEIONIZED WATER. (9765)

(NOTE:  KEEP THE SOLUTION COVERED AT ALL TIMES TO PREVENT ABSORPTION OF CO2.)

ADJUST THE PH OF THE SOLUTION TO 7.0 +- 0.2 PH UNITS WITH EITHER A WEAK SODIUM

HYDROXID (S2922) TO A WEAK HYDROCHLORIC ACID SOLUTION (H2505, (0.1 NORMAL NAOH

OR 0.1 NORMAL HCL).  DILUTE TO 1 LITER WITH DEIONIZED WATER AND STIR UNTIL

HOMOGENOUS.

NOTE:  IT IS NOT NECESSARY TO POST THE ACTUAL PH OF THE SOLUTION ON THE LABEL.

USE THE FOLLOWING EQUATION TO VERIFY 10.0 +- 0.1% W/V BARIUM CHLORIDE:

W/V% = (ACT GM B0360/ FINAL VOLUME IN LITERS / 10)

Revision:  0.0                           Date: 11/14/2019	Initial: STN

Expiration: 1 Year

Special posting instructions:  PUT CALCULATED W/V% BARIUM CHLORIDE TO 1 DECIMAL PLACE ON COA

Special Labeling Instructions: None, do  not put calculated value on label

Special Packaging:   Wear gloves (Nitrile) and goggles

Special Filtering Instructions: CAN BE PAPER FILTERED, GLASS WOOL OR Y STRAINER FILTERED.

Packaging:  10 liter cube

Retain: clear plastic

HMIS CODE: 200C

Packager Personal Protection: Gloves, goggles

Technician Personal Protection:  GLOVES, SPLASH GOGGLES, LABJACKET

Standardization: ADJUST  THE PH OF THE SOLUTION TO 7.0 +- 0.2 WITH A WEAK NAOH OR HCL SOLUTION USE ABOVE EQUATION TO VERIFY 20.0 +- 0.1% W/V BARIUM CHLORIDE

Appearance / Odor: Clear water white liquid   / odorless           Initial :_________

Disposal:  METALS WASTE ONLY!!! CONTAINS BARIUM, A REGULATED SUBSTANCE!!!

Revision 0.0 11/14/19: Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /