Customer Product Quality Requirements Form

Part Number: 8608		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

## 8608			SODIUM HYDROXIDE 0.01 NORMAL AQUEOUS SOLUTION

(N.I.S.T. TRACEABLE)

CLEAN TANK WITH DEIONIZED WATER (9765) BEFORE USE

WEAR GLOVES AND GLASSES WHILE MAKING THIS BATCH

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, ADD 0.8 GRAMS OF 50% W/W SODIUM HYDROXIDE (S2923, ADJUST BASED ON ACTUAL ASSAY) TO 800 MLS OF DEIONIZED WATER (9765) DILUTE TO 1 LITER WITH DEIONIZED WATER STIR UNTIL HOMOGENOUS.

REVISION:  2.0    DATE: 11/29/99                     INITIAL:___________.

EXPIRATION: 1 YEAR

SPECIAL PACKAGING INSTRUCTIONS: NONE

PACKAGING: CLEAR PLASTIC OR CUBES OR AMBER GLASS AT CUSTOMER REQUEST

HMIS CODE 200C

PERSONAL PROTECTION: GLOVES, GLASSES

STANDARDIZATION: QCSP7 TO 0.0100 +- 0.0005 NORMAL @20° C

APPEARANCE OF FINAL SOLN/ ODOR:  CLEAR WATER WHITE  ODORLESS LIQUID

DISPOSAL: GENERAL

REVISION ON 04/05/97 REVISED TO CHANGE RAW MATERIAL FROMPELLETS TO 50% NAOH

AND PUT QCSP STANDARDIZATION METHOD AND 2 YEARS EXP ON BATCH SHEET. STN

REVISION  2.0 11/ 29/99: REVISED BATCH SHEET AND BATCH CARD TO EXACT FORMULATION. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /