Customer Product Quality Requirements Form

Part Number: 2320		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

2320	Coper Sulfate

0.5 Molar Solution

Read SDS for raw materials and this batch sheet completely before beginning

Wear gloves (latex or Nitrile) and goggles when preparing this batch.

Rinse vessel with deionized water (9765) before use.

Rinse stir bar with deionized water and paper towel dry. Inspect for metal. Remove if present.

1-liter formulation approximate

For each liter of final solution, Adjust based on assay and dissolve 124.84 grams of reagent ACS grade Cupric Sulfate Pentahydrate (C8800) into about 800 mls of deionized water (9765). Once dissolved, dilute to 1 liter with deionized water (9765) and stir until homogenous.

Use the following equation to verify a 0.500 +- 0.005 Molar copper sulfate solution:

Molarity = (Act gm C8800 x assay) / 24968 / final volume in liters

Revision:  0.0	                             Date: 05-22-2018	Initial: STN

Expiration: 18 Months

Special Posting instructions:  Put calculated molarity to 3 decimal places on COA

Special Labeling instructions:  Do not put calculated molarity on label

Filtering instructions: This product may be glass wool or y-strainer filtered if needed

Special Packaging Instructions:   Wear gloves (nitrile or latex) and Goggles.

Packaging: 4 liter clear plastic, 4 liter cube, 10 liter cube

Retain: Clear plastic

HMIS Code:  100C

Technician Personal Protection:  Gloves (Nitrile or latex), goggles

Standardization:  Use above equation to verify 0.500 +- 0.005 Molar copper sulfate

Appearance/Odor:  Clear blue liquid / odorless     Initials:  _________

Disposal:  Metals waste only, contains copper

Revision 0.0 05-22-2018:  Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /