Customer Product Quality Requirements Form

Part Number: 7076		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

## 7076		Potassium Dichromate

0.06 Normal Certified Solution

Note:Potassium  Dichromate May intensify fire; oxidizer. Toxic if swallowed. Fatal if inhaled. May cause allergy or asthma symptoms or breathing difficulties if inhaled. May cause genetic defects. May cause cancer. May damage fertility or the unborn child. Causes damage to organs through prolonged or repeated exposure. Causes severe skin burns and eye damage. Harmful in contact with skin. May cause an allergic skin reaction. Potassium Dichromate is a hexavalent chromium containing compound. Read safety procedure before handling.

RINSE VESSEL WITH DEIONIZED WATER (9765)  BEFORE USE.  INSPECT STIR BAR FOR METAL AND REMOVE IF NECESSARY!!! WEAR GLOVES AND GOGGLES AND WEAR THE HEPA-FILTER RESPIRATOR CARTRIDGE WHILE HANDLING THE POTASSIUM DICHROMATEWHILE MAKING THIS BATCH.

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE 2.9422 GRAMS OF REAGENT ACS GRADE POTASSIUM DICHROMATE (P3785) INTO ABOUT 800 MLS OF DEIONIZED WATER .(9765)  DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

Revision:  0.0                           Date: 11/14/2019	 	Initial: STN

Expiration: 2 years

Special posting instructions:  Put normality on CoA to 4 decimal places

Special Labeling Instructions: Put normality on label to 4 decimal places. (0.0595-0.0605)

Special Packaging:   Wear gloves (Nitrile) and goggles

Special Filtering Instructions: None.

Packaging:  Clear plastic or cubes

Retain: clear plastic

HMIS CODE: 200C

Packager Personal Protection: Gloves, goggles

Technician Personal Protection:  Gloves, goggles, labjacket, Hepa filter respirator cartridge for (P3785).

Standardization: QCSP23 TO 0.0600 +- 0.0005 NORMAL @ 20°C

Appearance / Odor: Clear orange  / odorless          Initial :_________

Disposal:  METALS WASTE ONLY!!!! CONTAINS CHROMIUM!!!!

Revision 0.0 11/14/19: Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /