Customer Product Quality Requirements Form

Part Number: VUL010B		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL010                      DIMETHYLGLYOXIME

1% IN METHANOL

RINSE VESSEL WITH METHANOL (M2515) BEFORE USE.

WEAR GLOVES (AT LEAST ONE OF THE CHEMICALS IN THIS PRODUCT WILL PERMEATE NITRILE GLOVES. YOU MUST WEAR A BARRIER GLOVE UNDER A LARGER NITRILE GLOVE. REMEMBER, THE BARRIER GLOVE IS NOT A DISPOSABLE GLOVE.) AND GLASSES WHILE MAKING THIS BATCH.

1-LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION,  DISSOLVE 10.0 GRAMS OF DIMETHYLGLYOXIME (D5002) INTO ABOUT 800 MLS OF REAGENT ACS GRADE METHANOL (M2515).  DILUTE TO 1 LITER WITH REAGENT ACS GRADE METHANOL (M2515) AND STIR UNTIL HOMOGENOUS.

REVISION:  0.0                     DATE:  03/13/2014                     INITIAL:_STN__________.

EXPIRATION:   6 MONTHS

SPECIAL LABELING INSTRUCTIONS: NONE

FILTERING INSTRUCTIONS: NONE

SPECIAL PACKAGING INSTRUCTIONS:  GLOVES, GOGGLES

PACKAGING:  VUL010 =1 LITER AMBER GLASS, VUL010A=500ML AMBER GLASS

HMIS CODE :   320C FLAMMABLE

PERSONAL PROTECTION:  GLOVES, GLASSES, LABCOAT

STANDARDIZATION: NONE

APPEARANCE OF FINAL SOLN/ ODOR:    CLEAR WATER WHITE / METHANOL ODOR

DISPOSAL:   GENERAL WASTE

REVISION 0.0 03/13/14: CREATION DATE FOR BATCH CARD.  STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /