Customer Product Quality Requirements Form

Part Number: VUL243B		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL243                             NITRIC ACID

### 3 MOLAR AQUEOUS

RINSE VESSEL WITH DEIONIZED WATER (9765) BEFORE USE.

WEAR GLOVES (AT LEAST ONE OF THE CHEMICALS IN THIS PRODUCT WILL PERMEATE NITRILE GLOVES. YOU MUST WEAR A BARRIER GLOVE UNDER A LARGER NITRILE GLOVE. REMEMBER, THE BARRIER GLOVE IS NOT A DISPOSABLE GLOVE.) AND GLASSES WHILE MAKING THIS BATCH.

1-LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION,  ADD 189  MLS OF REAGENT ACS GRADE NITRIC ACID (N3805) INTO ABOUT 600 MLS OF DEIONIZED WATER (9765).  ONCE COOLED TO ROOM TEMERATURE, DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

Use the following equation to verify a nominal 3.00 +- 0.03 molar nitric acid solution:

Molarity = (Act mls N3805 / 63 / final volume in liters)

REVISION:  0.1                     DATE:  11-07-2019                     INITIAL:STN

EXPIRATION:   2 YEARS

SPECIAL POSTING INSTRUCTIONS: PUT CALCULATED MOLARITY TO 2 DECIMAL PLACES ON COA.

SPECIAL LABELING INSTRUCTIONS:  DO NOT PUT CALCULATED VALUE ON LABEL.

SPECIAL PACKAGING INSTRUCTIONS:  WEAR GLOVES AND GOGGLES, THIS SOLUTION IS EXPENSIVE.

PACKAGING:  VUL243 =  1 LITER AMBER GLASS , VUL243A = 4 LITER CUBE, VUL243B = 10 LITER CUBE VUL243C = 4 LITER AMBER PLASTIC

HMIS CODE :   300C CORROSIVE

PERSONAL PROTECTION:  GLOVES, GLASSES, LABCOAT

STANDARDIZATION: Use above equation to verify a nominal 3.00 +- 0.03 molar nitric acid solution

APPEARANCE OF FINAL SOLN/ ODOR:    CLEAR WATER WHITE / ODORLESS

DISPOSAL:   GENERAL

REVISION 0.0 01/12/01: CREATION DATE FOR BATCH CARD.  MG

Revision 0.1, 11-07-2019: added verification calculation and updated batch sheet to current format. STN.

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /