Customer Product Quality Requirements Form

Part Number: 7380		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

# 7380		POTASSIUM IODIDE 5% W/V AQUEOUS SOLUTION

Rinse vessel with deionized water before use.  Inspect stir bar for metal and remove if necessary!!! Wear gloves and glasses while preparing this batch.

## 1 LITER FORMULATION

For each liter of final solution, dissolve 50.0 grams of potassium iodide reagent acs grade (p4716) into about 700 mls of deionized water.(9765) add 1.0 grams of potassium hydroxide 45% w/w (7285) dilute to 1 liter with deionized water (9765) and stir until homogenous.

note: solution must be clear water white before release/ packaging.

Use the following equation to verify 5.00 +- 0.05% w/v potassium iodide:

w/v% = (act gms P4716 / final volume in liters / 10)

REVISION:  0.5	    DATE:  06/26/2019                     INITIAL: STN

EXPIRATION: 1 YEAR

SPECIAL POSTING INSTRUCTIONS: PUT CALCULATED W/V% POTASSIUM IODIDE TO 2 DECIMAL PLACES ON COA

SPECIAL LABELING INSTRUCTIONS: DO NOT PUT CALCULATED VALUE ON LABEL.

SPECIAL FILTERING INSTRUCTIONS: CAN BE GLASS WOOL, PAPER OR Y-STRAINER FILTERED.

SPECIAL PACKAGING INSTRUCTIONS: NONE

PACKAGING: 1 LITER AMBER PLASTIC; 20 LITER CUBE; 4 LITER AMBER PLASTIC;

4 LITER CUBE; 500 ML AMBER PLASTIC

HMIS CODE: 200B

PERSONAL PROTECTION: GLOVES, GLASSES

STANDARDIZATION: Use densitometer to verify density to 1.03490+/- 0.00148 (1.03342- 1.03638)

APPEARANCE OF FINAL SOLN/ ODOR: CLEAR WATER WHITE/ NONE

DISPOSAL: GENERAL

REVISION 0.1, 05/24/97:  revised to include addition of kho 45% to keep solution from yellowing.  STN

REVISION  0.2 11/ 23/99:reviewed and if necessary, revised batch sheet and batch card to exact formulation. STN

Revision 0.3 11/18/15: Added density specifications. DEJ

Revision 0.4, 08/03/2017: added verification calculation. STN

Revision 0.5: 06/26/2019: Updated density information using at least 8 data points. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /