Customer Product Quality Requirements Form

Part Number: 3948		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

- GLYCERIN

50% V/V SOLUTION

Rinse vessel with deionized water (9765) before use.

Inspect stir bar and remove any traces of metal before use

Wear gloves and goggles when making this batch.

1 Liter formulation

For each liter of final solution, add 500 mls of reagent ACS grade Glycerin (G7500) to about 400 mls of deionized water (9765) and stir until homogenous. Once homogenous, dilute to 1 liter with deionized water (9765) and stir until homogenous.

Use the following equation to verify 50.0 +- 0.1% v/v glycerin:

v/v% = (act mls glycerin / final volume in liters / 10)

Revision:  0.1                           Date: 05-19-2017	Initial: STN

Expiration: 2 Years

Special posting instructions: record calculated v/v% glycerin on COA

Special labeling instructions: Do not put calculated value on label

Special Packaging: Wear gloves and goggles

Special Labeling Instructions: None

Packaging: amber glass or hed-paks

HMIS CODE: 110C

Personal Protection:  Gloves, goggles

Standardization:  check and record density @25 deg C. Use above calculation to verify 50.0 +- 0.1% v/v glycerin

Appearance / Odor: Clear liquid  odorless    Initial:_________

Disposal:  Organic waste

Revision 0.0 08/06/07 Creation date for batch card. STN

Revision 0.1 05-1-2017: added verification calculation. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /