Customer Product Quality Requirements Form

Part Number: OXY538			    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXY538			Salt Acid Solution

Refer to each raw material SDS before handling the material for the first time.

Wear gloves (Nitrile), and goggles during production.

Prepare this product under the hood or wear a respirator while handling the hydrochloric acid.

Rinse vessel with deionized water (9765) before use.

Inspect stir bar and remove any traces of metal before use.

*If only 4 liters is ordered, prepare this solution volumetrically.

1 Liter formulation approximate

For each liter of final solution dissolve 239.60 grams of reagent ACS grade sodium chloride (S2675) into about 700 mls of deionized water (9765). Stir until homogenous. Next, for each liter of final solution measure out and add 20 mls of reagent ACS grade hydrochloric acid (H2505) and stir until homogenous. Dilute to 1 liter with deionized water and stir covered until homogenous.

Use the following equation to verify a nominal 4.10 +- 0.05 Normal sodium chloride and a 2.0 +- 0.1% v/v hydrochloric acid solution:

Normality NaCl = (Act gm S2675  / 58.44 / final volume in liters)

v/v% HCl = (Act mls H2505 / final volume in liters / 10)

Revision:  0.0                           Date: 11-15-2019	Initial: STN

Expiration: 18 months

Special posting instructions:  Put calculated normality sodium chloride to 2 decimal places and v/v% HCl to 1 on COA

Special Labeling Instructions: Do not put calculated values on label.

Special Packaging:   Wear gloves (Nitrile) and goggles, this solution has corrosive properties

Special Filtering Instructions: should not need filtering but can be glass wool filtered

Packaging:  10 Liter cube

Retain: clear pastic

HMIS CODE: 200C, corrosive liquid

Packager Personal Protection: Gloves, goggles

Technician Personal Protection:  Gloves, goggles, hood or respirator while handling hydrochloric acid.

Standardization: Use tabove equations to verify a nominal 4.10 +- 0.05 Normal sodium chloride and a 2.0 +- 0.1% v/v hydrochloric acid solution

Appearance / Odor: Clear water white liquid   / odorless           Initial :_________

Disposal:  General waste

Revision 0.0 11-15-19: Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /