Customer Product Quality Requirements Form

Part Number: 8665		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

8665	SODIUM HYDROXIDE

6.0 NORMAL AQUEOUS SOLUTION

CLEAN TANK WITH DEIONIZED WATER (9765) BEFORE USE.

WEAR GLOVES AND GOGGLES WHILE MAKING THIS BATCH.

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, SLOWLY ADD 480.0 GRAMS OF SODIUM HYDROXIDE

(S2923) 50% W/W (ADJUST BASED ON ACTUAL ASSAY) TO ABOUT 500 MLS OF DEIONIZED WATER (9765). ONCE COOLED TO ROOM TEMPERATURE, DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

REVISION:  2.4	    DATE: 07/23/2019                     INITIAL: STN

EXPIRATION:  2 YEARS

Special Posting Instructions: Put normality on CoA to 4 decimal places.

Special Labeling Instructions: Put normality on label to 4 decimal . (5.9700-6.0300)

PACKAGING: 10 LITER CUBE; 1 LITER CLEAR PLASTIC; 200 LITER BLUE DRUM; 20 LITER CUBE; 250 ML CLEAR PLASTIC; 4 LITER CLEAR PLASTIC; 500 ML CLEAR PLASTIC

Retain:Clear plastic

HMIS CODE:   300C, CORROSIVE LIQUID

PERSONAL PROTECTION: GLOVES, GOGGLES, SLEEVES

STANDARDIZATION:  QCSP7 TO 6.0000 ± 0.0300 NORMAL @ 20°C Use densitometer to verify the density of 1.21662+/- 0.00156 (1.21505- 1.21818)

APPEARANCE/ODOR: CLEAR WATER WHITE/NONE     Initials: __________

DISPOSAL:  GENERAL

REVISION 0.1, 04/02/99: REVISED TO CHANGE RAW MATERIAL TO 50% SODIUM HYDROXIDE FROM PELLETS AND PUT EXPIRATION TO 2 YEARS FROM 1 YEAR. STN

REVISION  2.0 11/ 29/99: REVISED BATCH SHEET AND BATCH CARD TO EXACT FORMULATION

REVISION 2.1, 12/08/08: ADDED STANDARDIZATION, PREVIOUSLY NOT STANDARDIZED. STN

Revision 2.2, 12/05/16: Added density specifications. DL\

Revision 2.3: 07/01/2019: Updated density information using at least 8 data points. STN

Revision 2.4, 07/23/19: Updated BOM notes to include posting, labeling, and retain instructions. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /