Customer Product Quality Requirements Form

Part Number: VUL242B		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL242                             NITRIC ACID

### 1.7% V/V AQUEOUS

RINSE VESSEL WITH DEIONIZED WATER (9765) BEFORE USE.

WEAR GLOVES (AT LEAST ONE OF THE CHEMICALS IN THIS PRODUCT WILL PERMEATE NITRILE GLOVES. YOU MUST WEAR A BARRIER GLOVE UNDER A LARGER NITRILE GLOVE. REMEMBER, THE BARRIER GLOVE IS NOT A DISPOSABLE GLOVE.) AND GLASSES WHILE MAKING THIS BATCH.

1-LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION,  ADD 17.0 MLS OF REAGENT ACS GRADE NITRIC ACID (N3805) INTO ABOUT 800 MLS OF DEIONIZED WATER (9765).  DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

Use the following equation to verify a 1.70 +- 0.01% v/v nitric acid solution:

V/V% Nitric Acid = (Act mls N3805 / Final volume in Liters / 10)

REVISION:  0.1                    DATE:  11-07-2019                     INITIAL: STN

SPECIAL POSTING INSTRUCTIONS: PUT CALCULATED V/V% NITRIC ACID TO 2 DECIMAL PLACES ON COA.

SPECIAL LABELING INSTRUCTIONS: DO NOT PUT CALCULATED VALUE ON LABEL.

EXPIRATION:   1 YEAR

SPECIAL PACKAGING INSTRUCTIONS:  NONE

PACKAGING:  VUL242 =  4 LITER CUBE, VUL242A = 4 LITER CLEAR PLASTIC

VUL242B = 10 liter cube

HMIS CODE :   200C, corrosive properties

PERSONAL PROTECTION:  GLOVES, GLASSES, LABCOAT

STANDARDIZATION: Use above equation to verify a 1.70 +- 0.01% v/v nitric acid solution

APPEARANCE OF FINAL SOLN/ ODOR:    CLEAR WATER WHITE / ODORLESS

DISPOSAL:   GENERAL

REVISION 0.0 01/12/01: CREATION DATE FOR BATCH CARD.  MG

Revision 0.1, 11-07-2019: added verification calculation and updated to current batch sheet format. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /