Customer Product Quality Requirements Form

Part Number: 8402		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

# 8402	                SILVER NITRATE 0.02 NORMAL AQUEOUS SOLUTION

N.I.S.T. TRACEABLE

RINSE VESSEL WITH DEIONIZED WATER (9765) BEFORE USE.

WEAR GLOVES AND GLASSES WHILE MAKING THIS BATCH.

1-LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, WEIGH OUT 3.3974 GRAMS OF SILVER NITRATE REAGENT ACS GRADE (S2121 ADJUST BASED ON ACTUAL ASSAY).  DISSOLVE IN ABOUT 800 MLS OF DEIONIZED WATER (9765). DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

REVISION:  0.4	 DATE:  09/19/2019                   INITIAL:__RI_________.

EXPIRATION: 1 YEAR

SPECIAL PACKAGING INSTRUCTIONS: NONE

PACKAGING: AMBER PLASTIC

HMIS CODE: 200C, OXIDIZER

PERSONAL PROTECTION: GLOVES, GLASSES

STANDARDIZATION: QCSP2 TO 0. 0200 +- 0.0005 NORMAL @ 20° C.Use the densitometer to verify the density of 1.00111+/- 0.00027 (1.00084- 1.00138)

APPEARANCE/ODOR: CLEAR WATER WHITE / NONE

DISPOSAL : METALS WASTE

REVISION: 0.0 06/11/00, ORIGINAL BATCH CARD CREATED   MG

REVISION 0.1 05-19-2015: ADDED QCSP78 CHECK

Revision 0.2: 06/30/15: added density information. STN

Revision 0.3: 06/27/2019: Updated density information using at least 8 data points. STN

Revision 0.4, 09/19/2019, removed qcsp78 due to low normality.  RI

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /